# Combined Evaluation Report: Franka Emika Panda and HEAL

Physical Grasp Pick-and-Place Evaluation

## 1. Overview

This report presents the results of a pick-and-place evaluation performed on two robots in MuJoCo: the Franka Emika Panda (7-DOF, parallel-jaw gripper) and the HEAL (6-DOF, three-jaw gripper). Both robots were tested with randomised cube poses. For the Franka, three inverse-kinematics methods (Mink, Damped Least Squares, and Quadratic Programming) were compared. For the HEAL, the Mink method was evaluated. Success is defined as the cube ending within 0.05 m (XY) of the tray centre after a full pick-place-release cycle.

## 2. Table Dimension Justification

### 2.1 Franka Emika Panda

Workspace analysis of the Franka (using the Mink library and site-based end-effector tracking) produced the following approximate reachable bounds in the task space:

- X: approximately 0.35 m usable range in front of the base

- Y: approximately 0.60 m lateral range

- Z: approximately 0.50 m vertical range above the table

The table was therefore sized to 0.35 m (X) × 0.60 m (Y) × 0.50 m (height), with the top surface at Z = 0.50 m. This places the entire working area inside the Franka’s reachable workspace while leaving a margin from the joint limits. The tray is centred on the table so that place targets remain well inside the dexterous region.

### 2.2 HEAL

Workspace analysis of the HEAL (sampling joint configurations and recording the end-effector site position) produced the following bounds:

- X: −0.747 m to 0.727 m

- Y: −0.739 m to 0.737 m

- Z: −0.425 m to 1.070 m

The table was sized to fit comfortably inside these bounds (approximately 0.50 m × 0.50 m working surface, top at Z ≈ 0.45 m). The three-jaw gripper is mounted near the end-effector site, and the table height was chosen so that the gripper can reach the cube with a near-vertical approach while remaining clear of joint limits. The tray is again centred so that place motions stay inside the high-manipulability region of the workspace.

## 3. Franka Emika Panda Results

Twenty episodes (seeds 2000–2019) were run with physical grasping. All three IK methods produced identical final offsets; the results are therefore reported once.

| Episode | Seed | Offset (m) | Result |
| --- | --- | --- | --- |
| 0 | 2000 | 0.039 | Success |
| 1 | 2001 | 0.976 | Fail |
| 2 | 2002 | 0.960 | Fail |
| 3 | 2003 | 0.028 | Success |
| 4 | 2004 | 0.160 | Marginal |
| 5 | 2005 | 0.947 | Fail |
| 6 | 2006 | 0.820 | Fail |
| 7 | 2007 | 0.946 | Fail |
| 8 | 2008 | 0.954 | Fail |
| 9 | 2009 | 0.727 | Fail |
| 10 | 2010 | 0.541 | Fail |
| 11 | 2011 | 0.725 | Fail |
| 12 | 2012 | 0.759 | Fail |
| 13 | 2013 | 0.872 | Fail |
| 14 | 2014 | 0.888 | Fail |
| 15 | 2015 | 0.947 | Fail |
| 16 | 2016 | 0.788 | Fail |
| 17 | 2017 | 0.645 | Fail |
| 18 | 2018 | 0.824 | Fail |
| 19 | 2019 | 0.768 | Fail |

### Summary

- Clear successes (offset ≤ 0.05 m): 2 / 20 (10 %)

- Marginal (0.05–0.20 m): 1 / 20

- Failures (> 0.20 m): 17 / 20 (85 %)

- Mean offset (all episodes): 0.71 m

The identical outcomes across Mink, DLS and QP show that the inverse-kinematics method is not the bottleneck. The dominant failure mode is unreliable physical contact with the parallel-jaw gripper.

## 4. HEAL Results

The HEAL was evaluated with the Mink controller and a three-jaw gripper. Because the arm is 6-DOF and the gripper is custom, the primary focus was on whether the end-effector could reach the cube and maintain a stable approach. Detailed per-episode offsets for the HEAL are recorded in the accompanying HEAL-specific notes; the key observation is that the three-jaw design improves lateral stability compared with a parallel-jaw gripper, but the lower number of arm degrees of freedom reduces the volume of the high-dexterity region.

Workspace coverage is larger in absolute terms than the Franka’s, yet the practical pick-and-place volume is constrained by the need to keep the three fingers clear of the table during the final descent.

## 5. Comparison and Conclusions

- Franka: higher dexterity (7-DOF) but low physical-grasp success with the parallel-jaw gripper under the current contact parameters.

- HEAL: three-jaw gripper offers better lateral retention, but the 6-DOF arm has a smaller high-manipulability volume for vertical approaches.

- IK method (Mink / DLS / QP) has negligible effect on Franka once contact is established.

- Table dimensions for both robots were chosen to lie inside the measured workspace while maximising the area available for randomised cube placement.

The main avenue for improvement on the Franka is contact modelling (friction and compliance). On the HEAL, further gains would come from refining the three-jaw finger geometry and approach height so that the fingers close without disturbing the cube.

End of Report